Circuit Design – Fixed Challenge 2019

Table of Contents

Circuit design – Main PCB  – Fixed Challenge 2019	4

Overview of the Circuit	4

Filtering	5

High pass	5

Low pass	6

Notch	6

Common mode chokes	7

Right leg driver	8

RF filters	8

Amplification	9

Operational amplifier	9

Instrumentation Amplifier	9

Circuit Amplification	10

Simulation	11

High pass filter	11

Low pass filter	12

Notch	13

Complete circuit for 1 channel	14

Other Components	15

Power supply	15

ADC	15

DC-to-DC converters	15

Protections	16

Schematic, layout and routing	17

Testing	19

Board cost	20

Circuit design – Secondary PCB  – Fixed Challenge 2019	21

Circuit Overview	21

Filtering	22

High pass filter	22

Low pass filter	22

Notch	23

Amplification	24

Instrumentation amplifier	24

Circuit amplification	24

Simulation	25

High Pass Filter	25

Low Pass Filter	26

Notch	27

Complete channel	28

Other components	29

ADC	29

Protection	30

Schematic, Layout and Routing	31

Testing	33

Board Cost	34

Annexes	35

Table 1 : List of components of the main PCB	35

Table 2 : List of components for secondary PCB	36

Reference	37

# Circuit design – Prototype PCB, version 3.0  – Fixed Challenge 2019

## Overview of the Circuit

PolyCortex is a neurotechnology student organization that partakes in the NeuroTechX Fixed and Open Challenge competitions. The electronic team tasked with designing the circuit and PCB for the acquisition of EEG signal (Fixed Challenge) works alongside the software team in charge of programming an interface to visualize the signal.

The circuits PolyCortex has design for this year’s fixed challenge are detailed in this document. This section covers the more ambitious prototype PCB. It is composed of 4 separate channels that can be wired to electrodes. Each channel includes filtering components (common mode chokes, RF filters), instrumentation amplifiers, various filters (high pass, low pass and notch) and a final amplification stage. Once the signal is treated and amplified, it is directed to the ADC which converts the signal from analog to digital in order to be forwarded to the interface for visualization. The circuit is powered by 9V batteries. However, the ADC is powered by ±2.5V provided by DC-to-DC converters included in the circuit. To insure the grounding standard of the board, a right leg driver (RLD) circuit configuration has been added to the board. Lastly, the subject wearing the electrodes is protected by a transient voltage suppressor (TVS).

The conception of the circuit was simulated with LTspice and implemented in the DipTrace software, which allows the creation of a schematic and layout of the PCB. It was printed by Labo Circuits Inc. and the components ordered from Digi-Key Electronics were welded by PolyCortex’s electronic team.

## Filtering

Due to their weak amplitudes, EEG signals are very susceptible to electromagnetic and common mode contamination. Additionally, EEG signals collected with electrodes may contain EMG information from the subject’s muscular activity and ECG signals from the polarizing cycles of heart cells. Is it thus important to filter the signal in order to isolate the frequency bands of interests for EEG analysis. The electroencephalogram is composed of 4 distinct waves ranging between 0.5 and 30Hz; the beta waves (13-30Hz), the alpha waves (8-13Hz), the theta waves (4-8Hz) and the delta waves (0.5-4Hz). PolyCortex therefore decided to filter outside of a bandwidth ranging from 0.3 to 35Hz to preserve all relevant EEG information.

Figure 1 : Frequencies of EEG waves

### High pass

The first filtering stage is a high pass Butterworth filter of the second order. The cutoff frequency f of such a filter is determined by the value of R2 and the value of C1  following the equation  f= 24*π*Ra*C1with R1=2 * R2 and C1 = C2. PolyCortex has chosen R1 = 75kΩ, R2 =37.4kΩ and C1 = C2 = 10µF, thus providing a cutoff frequency of 0.3Hz. The remaining resistors provide a gain to the filter, as explained in the Circuit Amplification section.

Figure 2 : High pass filter configuration (left : DipTrace schematic, right: theorical configuration)

### Low pass

To insure the cutting off of EMG signals and other noise, the Butterworth low pass filter has a cutoff frequency of 35Hz. The cutoff frequency of this filter is given by the following equation : f= 12*π*1R1R2C1C2 , referring to the right-side image of figure 3. In PolyCortex’s schematics (left-side of figure 3), these values have been set to R1 = 3.4Ωk, R2=60.4Ωk, C1= 0.1µF and C2= 1µF. Therefore, the cutoff frequency is 35Hz.

Figure 3 : Low pass filter configuration (left : DipTrace schematic, right: theorical configuration)

### Notch

When functioning near electrical power-lines, the electronic circuits will be contaminated by the mains hum, or electric hum, which is a noise associated with the alternating current of the power-line. In PolyCortex’s case, the fundamental frequency of the mains hum is 60Hz coming from Hydro-Quebec and has an approximated intensity of 30dB. It was considered wise to add a notch filter to the circuit to target this particular noise. For the chosen notch configuration, the cutoff frequency is given by the equation : f= 12*π*1R1R2C1C2 , with R3=R1+R2 and R4= R5=2* R3 (referring to the right-side of figure 4). To obtain a cutoff frequency centered around 60Hz with a gain of at least -30dB to eliminate the mains hum, PolyCortex chose values of R1 =  R2 = 5Ωk, R3 = 10Ωk, R4 = R5 = 20Ωk, C1 = 94nF and C2 = 3 µF. These values produce a gain of about              -36dB when simulated in LTspice (see Simulation section).

Figure 4 : DipTrace schematic for the notch filter (cut-off frequency of 60Hz)

### Common mode chokes

The circuit includes common mode chokes to eliminate a maximum of electromagnetic and radio frequency interferences from the power supply lines and other electronic devices. The common-mode current creates a magnetic field when passing through the coil that opposes any increase of its intensity. The magnetic field is thus blocking the common-mode current and passing differential current. PolyCortex chose CM4732V301R-10 by LAIRD, which works at a maximum rated current of 8,000mA and 30V.

Figure 5 : DipTrace schematic of common mode chokes (left) and equivalent circuit (right)

### Right leg driver

According to Texas Instruments, the common-mode rejection (CMR) is one of the most important parameters in ECG and EEG systems. Therefore, a right leg driver (RLD) circuit was added to further decrease the common-mode interference. The RLD circuit sets the user’s common-mode voltage in order to increase the effective CMR ratio of the circuit. To do so, it low-passes the common-mode voltage measured by the differential amplification stage. This method provides a grounding standard by preventing the lost of voltage due to the difference in impedance between the reference electrode on the subject and the circuit ground.

Figure 6 : Schematic for right leg driver

### RF filters

Radio frequency (RF) filters were added to the circuit to remove high frequency (MHz-GHz) signals originating from broadcast and wireless communication. The filtering of these frequencies is important considering they could affect the envelop of the output signal. PolyCortex uses Bourns Inc.’s EMI103T-RC filter for their good noise filtering properties, which also attenuate the mains hum with a factor of about -50dB.

Figure 7 : Configuration of RF filter (left : DipTrace schematic, right : functional diagram)

## Amplification

Electrical signals coming from the human body have weak amplitudes typically ranging from 1mV to 100mV. Specifically regarding the electroencephalogram (EEG), the voltage that can be measured at the surface of the brain is about 1-2mV whereas it decreases to microvolts (μV) when measured on the scalp with electrodes. Thus, acquiring and visualizing EEG signals requires amplification of approximatively 50,000. Such a gain allows the manipulation of the signals without saturating properly powered operational amplifiers present in the circuit.

### Operational amplifier

The op-amps used in the circuits are Texas Instruments’ trusted LM324. They were chosen for their built-in 4 operational amplifiers, their supply range of 3V to 32V and their typical common-mode rejection of -120dB.

Figure 8 : Functional diagram of LM324 amp-op

### Instrumentation Amplifier

The circuit to acquire EEG signals contain operational amplifiers placed after the electrodes to provide the signal with an initial gain before being filtered. The ins-amps used are Analog Devices’ AD8422, which are high performance, low power, rail-to-rail precision amplifiers.  Regarding the AD8422, the gain is determined by placing a single resistor RG across pin 2 and 3 (figure 9). PolyCortex decided the value of this resistor would be two times 250Ω, therefore inducing a gain of 40.6 (Gain = 1 + 19.8kΩ/RG). Furthermore, the datasheet suggests placing bypass capacitors (C3, C4, C5 and C6) as close as possible to each supply pin to regulate the supply tension.

Figure 9 : DipTrace Schematic and connection diagram of the in-op AD8422

### Circuit Amplification

To obtain the expected gain of ≈50,000, the different filtering layers can also be used to introduce a gain. The filters employed in this EEG circuit are second order Butterworth active filters where two resistors, Rf and Ri (R18 and R19 on figure 10), can be connected to the output signal without affecting the cutoff frequency. Consequently, the gain of the high pass and low pass filter is proportional to the ratio of these two resistors (Gain = 1 + Rf/Ri). During the design process, PolyCortex chose to introduce a gain of 8.9 in both filtering levels.

Figure 10 : High pass filter with a gain (G = 1 + R18/R19) of 8.9

After the signal has made its way through the amp-op and the filtering levels, it is amplified a final time with a non-inverting operational amplifier. For this configuration, the gain is once more proportional to the ratio of the chosen resistors (Gain = 1 + Rf/Ri) R30 and R31 on figure 11.

Figure 11 : Final amplification level with a gain of 12.8 (G = 1 + R30/R31)

The total gain produced by the cascading of the amp-op, the high pass and low pass filter and the non-inverter is thereby the multiplication of each individual gain, producing a final gain of ≈41100.

## Simulation

To insure the circuit behaves as it should, PolyCortex simulated every filtering stages with LTspice and tested the final amplification as well as the filtering capacities. In order to test the filters, a AC Analysis with 100 steps per decade, a start frequency of 0.1Hz and a stop frequency of 200Hz. This kind of analysis allows the visualizing of the circuit’s frequency response between the start and stop frequency and displays the Bode plot.

### High pass filter

Figure 12 : LTspice schematic of the high pass filter

Figure 13 : Bode plot of high pass filter with a cutoff frequency of 0.3Hz

### Low pass filter

Figure 14 : LTSpice schematic of low pass filter

Figure 15 : Bode plot of low pass filter with a cut off frequency of 35Hz

### Notch

Figure 16 : LTspice schematic of notch filter

Figure 17 : Bode plot of notch filter with a center frequency of 60Hz

### Complete circuit for 1 channel

To test the complete circuit of a channel, the Transient analysis was used with a stop time of 0.7 seconds. Such an analysis allows the visualisation of the non linear transition response of the circuit in the temporal domain, much like an oscilloscope would. The input signal is a sin wave with an amplitude of 50µV and a frequency of 10Hz. As seen in figure 19, the output signal has an amplitude of ≈2.05V, indicating a gain of ≈41000. It is important to note the slight offset, as the output sine wave is not centered around 0V. While testing the notch on a smaller specific PCB, PolyCortex observed the same offset in the output signal. The notch is therefore the likeliest cause of this offset.

Figure 18 : LTspice schematic of complete circuit of 1 channel

Figure 19 : Output signal of complete circuit of 1 channel

## Other Components

### Power supply

In previous version of the EEG acquisition circuit, the amp-ops were supplied with 5V and saturation was observed while gathering EEG data. Instead of decreasing the overall gain of the circuit, PolyCortex decided to increase the power supplying the circuit. The Vcc+ net of the circuit was thus set to 9V and the Vcc- to -9V since the board is powered with 9V batteries.

### ADC

The PCB created to acquire the EEG waves is made to be wired to an interface to visualize the signals. To insure the communication between the circuit and the interface’s program, the voltage of the four channels must be converted from analog to digital. Texas Instruments’ 24-bit ADS122C04 was selected for its 4-channel input and its high sampling rate of 2kSPS. Furthermore, this ADC uses the Delta-sigma analog-to-digital converting method, which pushes the noise to higher frequencies in order to increase its resolution. This ADC also helps eliminate common-mode noise by providing a typical CMRR of 110dB for a frequency of 60Hz (with DR=2kSPS).

Figure 20 : DipTrace schematic of the ADS122C04 ADC and functional diagram

### DC-to-DC converters

The ADC requires ±2.5V analog and digital supply to function. Therefore, a LT3483IS6 and a  LMZ12002TZ DC-to-DC converter were added to the circuit in order to transform the 9V from the battery to ±2.5V while also regulating the voltage input of the ADC. The LT3483 is commonly used to produce analog negative voltage outputs, while the LMZ12002TZ is used to carry out analog and digital output. Both were chosen for their input range (2.5V to 16V for LT3483IS6 and 4.5V to 20V for LMZ12002TZ), which largely accommodate the 9V of the battery, and their voltage output (-2.5V to -38V for LT3483IS6 and 0.8V to 6V for LMZ12002TZ) suitable to power the ADC.

Figure 21 : DipTrace schematic of DC-to-DC converter (top : LMZ12002TZ, bottom : LT3483IS6)

### Protections

Since PolyCortex will be testing the board on a human subject to acquire their EEG signals, it was considered wise to add a circuit protection component between the electrodes and the beginning of the circuit. Texas Instruments’ TPD4E1B06DCKR 4-channel bi-directional Transient Voltage Suppressor (TVS) diode array was chosen for its low leakage current (0.5nA) which insure the precision of analog measurements. It offers protection for currents exceeding 3.0A (8/20µS).

Figure 22 : DipTrace schematic of protection circuit TPD4E1B06DCKR and functional diagram

## Schematic, layout and routing

Before the PCB can be printed out, the circuit must be translated to a schematic. Then, the schematic is converted to a PCB layout, where the components must be placed and routed to one another. These steps were executed with the DipTrace software, which offers a schematic capture editor, a PCB layout editor, a component editor and a pattern editor module. The component and pattern editor tools are used to incorporate custom components that might not be included in DipTrace libraries.

Figure 23 : DipTrace schematic for 4-channel circuit

The PCB PolyCortex created has 4 layers : the top one contains the pads on which the components are wielded and wires, the middle layers consists of the ground and power supply, and the bottom layer contains additional wiring. A copper plating was placed on top to surround the components and wires to further isolated them and reduce parasitic interference between the channels.

When doing the layout, PolyCortex took into consideration the size of the board, which could not exceed a self placed limit of 20cm by 20cm, and the proximity of the components. The components were placed as close to each other as possible to minimize signal quality degradation. Furthermore, the analog and digital components were separated to avoid interference. While routing the circuit, the 90° angles were carefully avoided to prevent current concentration. Long parallel wire line were also avoided to stop parasitic impedance from contaminant the signals.

Figure 24 : DipTrace layout of the finished PCB

The completed PCB was exported in gerber files and sent to Labo Circuits Inc. for printing.

## Testing

PolyCortex has not completed the testing phase of the prototype PCB. Due to the circuit’s complexity and the highly optimized layout, soldering the components onto the board was much longer and much harder then anticipated.  Key components, such as the DC-to-DC converters and common mode chokes, required more advanced soldering techniques not previously explored by PolyCortex. Thus, the PCB will be completed over the summer of 2019, using hot air soldering and reflow ovens. Testing this prototype PCB is important to further the team’s comprehension of designing more complex circuits and understanding of the limitations of electronics. The prototype PCB includes many testing points (TP1 through TP22) that’ll help PolyCortex identify and rectify the problems with the PCB in order to submit it in a future NeuroTechX competition.

The filtering stages that were tested seemed to cause unwanted saturation of the amp-ops visible from the different testing points of the board. PolyCortex believes this observation is due to gains added to the filtering levels.

## Board cost

A list of the 207 components of the circuit is annexed to this document. The total cost of the components, which were ordered on Digi-Key Electronics, is 157,06 CAN$. The PCB was printed by Labo Circuits Inc. for a fix cost of 225 CAN$ and 57,16$ per copy. The board cost can therefore be evaluated to 282,16 CAN$. The total cost of the board and the component is 439,21 CAN$ (329,67 US$).

## Encountered difficulties and limitations

The most obvious limitation of the prototype PCB is that it has not yet been fully tested and isn’t ready to acquire EEG waves from a subject. Furthermore, the size of its components 0603 (metric) and the proximity of the pads on the layout make the board considerably hard to solder by hand. PolyCortex has encountered problems soldering components on pads too close to each other, as the melted tin would sometimes touch other pads and create shorts.

## Lessons learnt

The prototype PCB has a much greater complexity than any board previously attempted by PolyCortex. Although it is not yet fully functional, this ambitious PCB has a lot of potential and lessons to teach. First of all, the difficulties soldering the PCB brought PolyCortex to reconsider using 0603 components for future boards. A bigger size of 1206 (metric) will most definitely be used in future designs, since there are no particular restrictions on the overall size and weight of the PCB. Using components easier to solder would also simplify the reproducibility of the board and minimize soldering errors. Since PolyCortex is a team that values inclusion and teaching of new skills, easier soldering would allow more members to participate in assembling the PCB. Additionally, a special attention will be paid to the distance between the pads when laying out the components. Further conclusions will arise from testing the board. PolyCortex is looking to determinate the effect of components like RF filters and common mode chokes on the quality of the signal and the artefact removal. Moreover, the  24 bits ADC used on this PCB is more powerful and better suitable for EEG application than ADC used on previous cards and will likely help PolyCortex acquire more precise data in future versions of the PCB.

# Circuit design – Secondary PCB, version 2.2  – Fixed Challenge 2019

## Circuit Overview

The circuit presented previously is considerably more complex than the minimum required to acquire EEG waves. Therefore, PolyCortex chose to develop a secondary PCB to account for the potential mistakes in the prototype PCB and ensure the delivery of a functional board. This secondary PCB contains the strict minimum to filter, amplify and convert the signals from analog to digital, while still insuring the safety of the subject to which the electrodes are connected. The circuit separates the analog and the digital portion, which are respectively powered by 9V batteries and by an Arduino microcontroller (3.3V). The analog portion includes 4 channels composed of an instrumentation amplifier, three stages of filtering and a final amplification stage. The digital portion includes the ADC component to insure the transfer of the data onto the interface. Furthermore, the board has two separate outputs for digital and analog data.

## Filtering

### High pass filter

During preliminary testing of the prototype card, the PolyCortex team noticed unexpected saturation behavior between the different filtering stage. It was thus decided to remove the resistors responsible for the gain of the filters (see figure 10 and 11), as they can modify the frequency response and can degrade the signal if the gain is too high. The high pass filter used for this circuit is the same Butterworth 2nd order configuration, but it doesn’t provide any amplification gain to the signal. The cutoff frequency f of such a filter is determined by the value of Ra and the value of Ca following the equation : f= 24*π*Ra*Ca , with Ra= 2 * Rb and Ca= Cb. PolyCortex has chosen Ra = 75kΩ, Rb =37.4kΩ and Ca = Cb = 10µF, thus providing a cutoff frequency of 0.3Hz.

Figure 25 : Secondary PCB high pass filter configuration (left : DipTrace schematic, right: theorical configuration)

### Low pass filter

Similarly to the high pass filter, the resistors responsible for the gain of the lowpass filter were removed from the configuration. The cutoff frequency of this filter is given by the following equation : f= 12*π*1R1R2C1C2 , referring to the right-side image of figure 26. In PolyCortex’s schematics (left-side of figure 3), these values have been set to R1 = 3.4Ωk, R2=60.4Ωk, C1= 0.1µF and C2= 1µF. Therefore, the cutoff frequency is 35Hz.

Figure 26 : Secondary PCB  low pass filter configuration (left : DipTrace schematic, right: theorical configuration)

### Notch

The notch used for this circuit is the same as the one designed for the prototype PCB (see figure 4), and therefore also has a narrow bandwidth cutting the 60Hz frequency. For this specific configuration, the cutoff frequency is given by the equation : f= 12*π*1R1R2C1C2, with R3=R1+R2 and R4= R5=2* R3 (referring to the right-side of figure 4). To obtain a cutoff frequency centered around 60Hz with a gain of at least -30dB to eliminate the mains hum, PolyCortex chose values of R1 =  R2 = 5Ωk, R3 = 10Ωk, R4 = R5 = 20Ωk, C1 = 0.12µF and C2 = 2.2µF.

Figure 27 : Secondary PCB  notch  filter configuration (left : DipTrace schematic, right: theorical configuration)

## Amplification

Since the resistors providing a gain on each filtering stages were removed, the amplification provided by the instrumentation amplifier and the last amplification stage needed to be increased. Like the prototype PCB, the operational amplifiers used are Texas Instruments’ LM324 (see Operational amplifier section of prototype PCB and figure 8). Furthermore, the instrumentation amplifiers used for this circuit are also the AD8422 (see figure 9).

### Instrumentation amplifier

The in-op configuration used in the secondary card differs from the prototype circuit regarding the resistor used and the gain produced. The gain, which is given by Gain = 1 + 19.8kΩ/RG,  was increased by using a single 50Ω RG resistor instead of two 250Ω resistors. Thus, the instrumentation amplifier is responsible for a gain of  about 398. The bypass capacitors used in the prototype PCB were kept to regulate the supply voltage of the in-op.

Figure 28 : DipTrace Schematic for secondary card and connection diagram of the in-op AD8422

### Circuit amplification

Similarly to the prototype PCB, this circuit contains a final amplification stage located after the filters. It provides a gain because of its non-inverter op-amp configuration. For this configuration, the gain is once more proportional to the ratio of the chosen resistors (Gain = 1 + Rf/Ri) R15 and R16 on figure 29. To increase the gain compared to the prototype PCB, PolyCortex chose to use R15=400kΩ and R16=3.4kΩ, therefore providing a gain of ≈119.

Figure 29 : Diptrace schematic of non-inverter final amplification stage of secondary PCB

The total gain of the circuit is therefore ≈47 362 (398 * 119).

## Simulation

Using the LTspice software, the filters were individually simulated using the same method as the prototype PCB and a Bode plot was produced to verify the cutoff frequencies.

### High Pass Filter

Figure 31 : LTspice schematic of high pass filter for secondary PCB

Figure 32 : Bode plot of high pass filter with cutoff frequency of 0.3Hz  for secondary PCB

### Low Pass Filter

Figure 33 : LTspice schematic of low pass filter for secondary PCB

Figure 34 : Bode plot of low pass filter with cutoff frequency of 35Hz  for secondary PCB

### Notch

Figure 35 : LTspice schematic of notch filter for secondary PCB

Figure 36 : Bode plot of notch filter for secondary PCB

### Complete channel

The circuit for a complete EEG acquisition channel was simulated with an input sinusoidal source with an amplitude of 100µV peak-to-peak. The output signal is roughly 4.6V peak-to-peak, indicating a over gain of ≈46 000, which is coherent with the theorical gain of 47 362. The different between the theory and the simulation lies in the tolerance of the resistors. A slight offset is visible on the output signal, as it is not completely centered around 0V like it was originally, which is most likely due to the notch filter used. However, this offset will not affect the visualisation of the signals, as the peak-to-peak amplitude is the feature of interest.

Figure 37 : LTspice schematic of complete channel circuit for secondary PCB

Figure 38 : Output signal of complete circuit of 1 channel for secondary PCB

## Other components

### ADC

PolyCortex chose to resort to an ADC model that had been tested by the team with the printing of a PCB containing only the ADC and its corresponding circuit. Thus, instead of using the Delta-Sigma ADS112C04 of the prototype PCB, the secondary PCB uses Texas Instruments’ ADS7828. This ADC is slightly less performant, as it uses a sampling method over 12 bits instead of the Delta-Sigma converting process over 24 bits of the ADS112C04. The ADS7828 has a built in asynchronous clock, an 8-channel multiplexer (MUX) and a sample-and-hold amplifier. It supports the I2C interface and can be set to standard, fast and high-speed modes. In order to communicate with the interface PolyCortex has created to visualise the EEG signal, the ADC’s output is connected to an Arduino microcontroller. The Arduino also supplies the ADC with 3.3V, as the ADS7828 has an input range of 2.3V to 5V.

Figure 39 : DipTrace schematic of the ADS122C04 ADC and functional diagram

#### Communication between the ADC and Arduino details

(feel free de delete la photo d’arduino ahahha elle est plus là pour attirer ton attention, quoi que ca serait nice de mettre une photo de la petite carte hooked up au arduino)

### Protection

To prevent the subject from being electrocuted by leakage electricity, the secondary card also uses  Texas Instruments’ TPD4E1B06DCKR 4-channel bi-directional Transient Voltage Suppressor (TVS) diode array. It has a low leakage current (0.5nA), which insure the precision of analog measurements and it offers protection for currents exceeding 3.0A (8/20µS).

Figure 40 : DipTrace schematic of protection circuit TPD4E1B06DCKR and functional diagram

## Schematic, Layout and Routing

The PCB for this circuit was routed in Diptrace the same way the prototype PCB was. However, PolyCortex noticed the components on the prototype PCB were very close to each other, making the board hard to weld. Thus, the components on the secondary board were placed further apart while still respecting the maximum dimensions of 20cm by 20cm. The PCB itself, due to its simpler nature, was laid out and printed on only two layers (the prototype PCB has four). The components lie on the top layer, which is related to the bottom layer through static vias. Both top and bottom layers are coved by an extra coating of copper to provide the circuit’s ground, isolate the components and reduce interference between the channels. The input and output pin locations were placed as much as possible on the edges of the board to facilitate access while still reducing route lengths.

Figure 41 : Diptrace schematic of complete 4 channels

Figure 42 : Diptrace layout and routing for secondary PCB

The PCB was printed with PCBway, reducing the cost, but unfortunately the quality, compared  to printing with Labo Circuit. PolyCortex also ordered the stencil used to weld the components onto the PCB with a soldering reflow oven.

## Testing

## Board Cost

A list of the 138 components of the circuit is annexed to this document. The total cost of the components ordered on Digi-Key Electronics is 102,87 CAN$. Five copies of the PCB were printed by PCBways for a fix cost of 141 CAN$. The board cost can therefore be evaluated to 282,16 CAN$. The total cost of the board with the component is 243,87 CAN$ (183,10 US$). The secondary board is much cheaper than the prototype due to the simplicity of its circuit and the lesser quality of the printed PCB.

# Annexes

## Table 1 : List of components of the prototype PCB

| Component | Value | Pattern | Quantity | $/u | Total $ |
| --- | --- | --- | --- | --- | --- |
| ADC | - | ADS122C04 | 1 | 9,93 | 9,93 |
| Capacitor | 0.1u | CAP_0603 | 43 | 0,0558 | 2,3994 |
| Capacitor | 1u | CAP_0603 | 6 | 0,15 | 0,9 |
| Capacitor | 10u | CAP_0603 | 9 | 0,21 | 1,89 |
| Capacitor | 0.12u | CAP_0603 | 4 | 0,33 | 1,32 |
| Capacitor | 2.2u | CAP_0603 | 4 | 0,154 | 0,616 |
| Capacitor | 0.022u | CAP_0603 | 2 | 0,073 | 0,146 |
| Capacitor | 100u | CAP_1210 | 1 | 0,949 | 0,949 |
| Capacitor | 4.7u | CAP_0603 | 6 | 0,15 | 0,9 |
| Capacitor | 0.47u | CAP_0603 | 1 | 0,12 | 0,12 |
| Capacitor | 5pF | CAP_0603 | 1 | 0,061 | 0,061 |
| Common mode choke | - | CM4732V301R-10 | 1 | 4,43 | 4,43 |
| CON | - | CON6M | 14 | 0,068 | 0,94 |
| DCDC Converter | - | LMZ12002 | 1 | 7,69 | 7,69 |
| DCDC Converter | - | LT3483IS6 | 1 | 6,81 | 6,81 |
| Diode | - | PMEG3002AEB | 1 | 0,49 | 0,49 |
| Inductance | 4.7uH | LQH2MCN100K02L | 1 | 0,38 | 0,38 |
| Inductance | - | Ferrite-200 | 2 | 0,079 | 0,158 |
| Instrumentation amp | - | AD8422 | 4 | 8,81 | 35,24 |
| Operational amp | - | LM324 | 10 | 0,295 | 2,95 |
| Potentiometer | 500-1M | 201XR | 1 | 0,79 | 0,79 |
| Protection | - | TPD4E1B0 | 2 | 0,736 | 1,472 |
| Resistor | 26.7k | RES_0603 | 8 | 0,53 | 4,24 |
| Resistor | 3.4k | RES_0603 | 16 | 0,53 | 8,48 |
| Resistor | 250 | RES_0603 | 8 | 2,15 | 17,2 |
| Resistor | 37.4k | RES_0603 | 4 | 0,15 | 0,6 |
| Resistor | 75k | RES_0603 | 4 | 1 | 4 |
| Resistor | 60.4k | RES_0603 | 4 | 0,53 | 2,12 |
| Resistor | 20k | RES_0603 | 8 | 0,1172 | 0,9376 |
| Resistor | 10k | RES_0603 | 9 | 0,15 | 1,35 |
| Resistor | 5k | RES_0603 | 8 | 3,07 | 24,56 |
| Resistor | 40k | RES_0603 | 4 | 0,839 | 3,356 |
| Resistor | 1k | RES_0603 | 4 | 0,15 | 0,6 |
| Resistor | 32.4k | RES_0603 | 2 | 0,1172 | 0,2344 |
| Resistor | 2.5k | RES_0603 | 1 | 1,217 | 1,217 |
| Resistor | 11.8k | RES_0603 | 1 | 0,3 | 0,3 |
| Resistor | 2.1k | RES_0603 | 1 | 0,39 | 0,39 |
| Resistor | 250k | RES_0603 | 1 | 0,8761 | 0,8761 |
| Resistor | 10 | RES_0603 | 1 | 0,54 | 0,54 |
| RF Filter | - | EMI T filter | 6 | 0,483 | 2,898 |
| Supply battery | 9V | - | 1 | 2,572 | 2,572 |
| TOTAL | - | - | 207 | - | 157,06 |

## Table 2 : List of components for secondary PCB

| Componant | Value or Model | Pattern | Quantity | Price $/u | Total |
| --- | --- | --- | --- | --- | --- |
| ADC | ADS7828 | ADS7828 | 1 | 11,66 | 11,66 |
| Capacitor | 0.12u | CAP_0603 | 4 | 0,33 | 1,32 |
| Capacitor | 0.1u | CAP_0603 | 31 | 0,0558 | 1,7298 |
| Capacitor | 10u | CAP_0603 | 8 | 0,21 | 1,68 |
| Capacitor | 1u | CAP_0603 | 5 | 0,15 | 0,75 |
| Capacitor | 2.2u | CAP_0603 | 4 | 0,154 | 0,616 |
| CON | Male | - | 22 | 0,0671429 | 1,4771438 |
| Instrumentation amp | AD8422 | AD8422 | 4 | 8,81 | 35,24 |
| Operational amp | LM324 | LM324 | 5 | 0,295 | 1,475 |
| Protection | TPD4E1B06DCKR | TPD4E1B0 | 1 | 0,739 | 0,739 |
| Resistor | 10k | RES_0603 | 4 | 0,15 | 0,6 |
| Resistor | 20k | RES_0603 | 8 | 0,1172 | 0,9376 |
| Resistor | 2k | RES_0603 | 2 | 0,53 | 1,06 |
| Resistor | 3.4k | RES_0603 | 8 | 0,53 | 4,24 |
| Resistor | 37.4k | RES_0603 | 4 | 0,15 | 0,6 |
| Resistor | 400k | RES_0603 | 4 | 0,511 | 2,044 |
| Resistor | 5 | RES_0603 | 1 | 0,24 | 0,24 |
| Resistor | 50 | RES_0603 | 4 | 0,16 | 0,64 |
| Resistor | 5k | RES_0603 | 8 | 3,07 | 24,56 |
| Resistor | 60.4k | RES_0603 | 4 | 0,53 | 2,12 |
| Resistor | 75k | RES_0603 | 4 | 1 | 4 |
| Supply | 9V | - | 2 | 2,572 | 5,144 |
| TOTAL | - | - | 138 | - | 102,87 |

# Reference